| Factors of practicability | Results | Observations |
| --- | --- | --- |
| Time Taken | Black High: 10mins 28 secs Black Mid: 3mins 46 secs Black Low: 2mins 59secs Exp -0.3 White High: 3mins 58secs White Mid: 2mins 15secs White Low: 4mins 50secs | 1st print took extra time to work out how to successful capture the print. The tripod and holding the mirror were unsuccessful. The mirror had to be taped to the door in order to capture the print, while using the flash. Diffused reflections cause light to be reflected in the mirror. |
| One or two hands needed | Two hands | Handheld camera and flash |
| Use of additional equipment e.g. mirror | Mirror and tape | Mirror taped to door to gain reflection of print to photograph. If unable to tape mirror could not capture print. |
| Lighting needed e.g. flash or torch | Flash needed, sync cord | Flash angled towards the actual print not the mirror and was held as far back as possible. But difficult due to limited working space (5cm). Used 2 second timer next time round to allow to set flash in a better position. |
| Ease of viewing (e.g. attached viewing screen or additional viewing screen) | Attached viewing screen to camera | Using the viewfinder was the better option while capturing this fingermark. |
| Storage of images (SD card or device storage) | SD Card |  |
| Successful capture of fingermark including scale and label. | Yes | Fingermark was captured clearly. Linear scale and label can be included in image. But there is distortion to the print as its reflection on the mirror was captured not the actual print. |
| Security of images | SD card manual transfer of files into secure locations | Ensure SD card is kept in a secure location at all times and images are transferred to appropriate locations as soon as possible. |

DSLR/ Standard Methods Situation 1: Behind a door handle

Camera Settings: Manul mode, f/22, 1/125th shutter speed, exp 0 or -0.3, ISO 100 and evaluative metering.

DSLR/ Standard Methods Situation 2: On top of a fence

| Factors of practicability | Results | Observations |
| --- | --- | --- |
| Time Taken | Black High: 7 minutes 17 seconds (16) Black Mid: 5mins 15secs (22) Black Low: 7 mins 56secs (22) White High: 7 mins 3 secs White Mid: 4min 17 secs White Low: 6mins 22secs | Difficult to focus camera, handheld. Hand cramps and gets sore due to holding the heavy equipment for extended amount of time. Used 2 second timer next time round to allow to set flash in a better position. |
| One or two hands needed | Two hands | Difficult to manage flash and camera without touching apparatus and to hold in position for successful capture. |
| Use of additional equipment e.g. mirror | Stepladder if available | Due to the limitations of the researcher’s height, a stepladder was needed to capture the print. Capturing the print without the stepladder was conducted beforehand and was unsuccessful. |
| Lighting needed e.g. flash or torch | Flash needed, sync cord | Flash held at a 45-degree angle but couldn’t hold in far enough back due to managing the equipment to be able to capture the print. Used 2 second timer next time round to allow to set flash in a better position. |
| Ease of viewing (e.g. attached viewing screen or additional viewing screen) | Attached viewing screen to camera | The screen could be flipped out in order to see it while capturing the photo. |
| Storage of images (SD card or device storage) | SD Card |  |
| Successful capture of fingermark | Yes | Fingermark was captured clearly. Linear scale and label can be included in image |
| Security of images | SD card manual transfer of files into secure locations | Ensure SD card is kept in a secure location at all times and images are transferred to appropriate locations as soon as possible. |

Camera Settings: AV mode and manual mode, exp 0, ISO 100, f/stop 16 and 22, spot and evaluative metering. 1/250th or 1/125th shutter speed.

DSLR/ Standard Methods Situation 3: Outside on a windowsill

| Factors of practicability | Results | Observations |
| --- | --- | --- |
| Time Taken | Black High: 10mins 01 secs Black Mid: 2mins 47secs Black Low: 3mins 56secs White High: 1min 30secs White Mid: 1min 32secs White Low: 1min 39secs | Hand cramping due to holding camera out with one hand and for a long time. 1st print took extra time to work out how to successful capture the print. |
| One or two hands needed | Two Hands | Heavy weight holding camera with one hand. |
| Use of additional equipment e.g. mirror | No |  |
| Lighting needed e.g. flash or torch | Flash needed, sync cord | Flash held at a 45-degree angle, flash at 0 exp too bright and flash at -0.3 exp too dim. To be able to hold flash there is a slight shadow of the linear scale. Used 2 second timer next time round to allow to set flash in a better position. |
| Ease of viewing (e.g. attached viewing screen or additional viewing screen) | Attached viewing screen to camera | The screen could be flipped out in order to see it while capturing the photo. |
| Storage of images (SD card or device storage) | SD Card |  |
| Successful capture of fingermark | Yes | Fingermark was captured clearly. Linear scale and label can be included in image |
| Security of images | SD card manual transfer of files into secure locations | Ensure SD card is kept in a secure location at all times and images are transferred to appropriate locations as soon as possible. |

Camera Settings: ISO 100, shutter speed 1/125th, f/stop 16, exp -0.3

DSLR/ Standard Methods Situation 4: Under a handrail

| Factors of practicability | Results | Observations |
| --- | --- | --- |
| Time Taken | Black High: Black Mid: Black Low: White High: White Mid: White Low: |  |
| One or two hands needed |  |  |
| Use of additional equipment e.g. mirror |  |  |
| Lighting needed e.g. flash or torch |  |  |
| Ease of viewing (e.g. attached viewing screen or additional viewing screen) |  |  |
| Storage of images (SD card or device storage) |  |  |
| Successful capture of fingermark |  |  |
| Security of images |  |  |

Camera Settings:

DSLR/ Standard Methods Situation 5: Inside an evidence item (Safe)

| Factors of practicability | Results | Observations |
| --- | --- | --- |
| Time Taken | Black High: Black Mid: Black Low: White High: White Mid: White Low: |  |
| One or two hands needed |  |  |
| Use of additional equipment e.g. mirror |  |  |
| Lighting needed e.g. flash or torch |  |  |
| Ease of viewing (e.g. attached viewing screen or additional viewing screen) |  |  |
| Storage of images (SD card or device storage) |  |  |
| Successful capture of fingermark |  |  |
| Security of images |  |  |

Camera Settings:

| Time Taken | Black High:1min 40secs Black Mid: 2mins Black Low: 2mins White High: 2mins White Mid: 2mins 29 secs White Low: 50secs ` Got to ensure that print is in centre before lifting so print isn’t cut off. | When print is lifted the linear scale sticker comes with it. Not all the print was lifted. Time, equipment and hands is how the lift is taken not photographing the lift. Information included on how the lift was photographed. The reflective surface if the lift causes issues when photographing, the air bubbles and tape surface as reflect back as well as the print. |
| --- | --- | --- |
| One or two hands needed | Two hands | One to lift the other for the mirror |
| Use of additional equipment e.g. mirror | Mirror | A mirror was needed to see the print before lifting. Would not lift print blind. |
| Lighting needed e.g. flash or torch | Flash needed, sync cord | How lift is photographed. Flash held at a 45-degree angle and held far back to gain even lighting. |
| Ease of viewing (e.g. attached viewing screen or additional viewing screen) | Attached viewing screen to camera |  |
| Storage of images (SD card or device storage) | SD Card |  |
| Successful capture of fingermark including scale and label. | Yes | Fingermark was captured clearly. Linear scale and label can be included in image |
| Security of images | SD card manual transfer of files into secure locations | Ensure SD card is kept in a secure location at all times and images are transferred to appropriate locations as soon as possible. |

Lift Standard Methods Situation 1: Behind a door handle

Camera Settings: Manual mode, exp -0.3, ISO 100, f/stop 22, evaluative metering, 1/125th shutter speed.

The lift was captured with a DSLR as its standard practice. Capture a photo handheld to replicate capturing the lift at a scene and on a copy stand to replicate the lift being captured at the lab. After talking with supervisor just photographing on a copy stand is enough reminding photos captured this way.

Lift Standard Methods Situation 2: On top of a fence

| Factors of practicability | Results | Observations |
| --- | --- | --- |
| Time Taken | Black High: 1min 14 secs Black Mid: 2 min 11 secs Black Low: 1 min 6secs White High: 1min 27secs White Mid: 2 mins 36 secs White Low: 1min 58secs | When print is lifted the linear scale sticker comes with it. Not all the print was lifted. Time is how long the lift takes not photographing the lift. Information included on how the lift was photographed. The reflective surface if the lift causes issues when photographing, the air bubbles and tape surface as reflect back as well as the print. |
| One or two hands needed | Two hands |  |
| Use of additional equipment e.g. mirror | Stepladder needed to view print in order to lift it correctly. | If there is no ladder couldn’t lift the prints as would not lift the print blind. |
| Lighting needed e.g. flash or torch | Flash needed, sync cord | Flash held at a 45-degree angle and held back to gain even lighting. |
| Ease of viewing (e.g. attached viewing screen or additional viewing screen) | Attached viewing screen to camera |  |
| Storage of images (SD card or device storage) | SD Card |  |
| Successful capture of fingermark | Yes | Fingermark was captured clearly. Linear scale and label can be included in image |
| Security of images | SD card manual transfer of files into secure locations | Ensure SD card is kept in a secure location at all times and images are transferred to appropriate locations as soon as possible. |

Notes: The lift was captured with a DSLR as its standard practice. Capture a photo handheld to replicate capturing the lift at a scene and on a copy stand to replicate the lift being captured at the lab. After talking with supervisor just photographing on a copy stand is enough reminding photos captured this way.

The plastic tab that is peeled off is leaving markings which are now apart of the lifted print.

Camera Settings: AV mode and manual mode, exp 0, ISO 100, f/stop 16 and 22, spot and evaluative metering, 1/125th shutter speed.

Lift Standard Methods Situation 3: Outside on a windowsill

| Factors of practicability | Results | Observations |
| --- | --- | --- |
| Time Taken | Black High: 44secs Black Mid: 1min 5secs Black Low: 40secs White High: 57secs White Mid: 38secs White Low: 57secs | When print is lifted the linear scale sticker comes with it. Not all the print was lifted. Time is how long the lift takes not photographing the lift. Information included on how the lift was photographed. The reflective surface if the lift causes issues when photographing, the air bubbles and tape surface as reflect back as well as the print. |
| One or two hands needed | Two hands |  |
| Use of additional equipment e.g. mirror |  |  |
| Lighting needed e.g. flash or torch | Flash needed, sync cord | Flash held at a 45-degree angle and held back to gain even lighting. |
| Ease of viewing (e.g. attached viewing screen or additional viewing screen) | Attached viewing screen to camera |  |
| Storage of images (SD card or device storage) | SD Card |  |
| Successful capture of fingermark | Yes | Fingermark was captured clearly. Linear scale and label can be included in image |
| Security of images | SD card manual transfer of files into secure locations | Ensure SD card is kept in a secure location at all times and images are transferred to appropriate locations as soon as possible. |

Lift Standard Methods Situation 4: Under a handrail

| Factors of practicability | Results | Observations |
| --- | --- | --- |
| Time Taken | Black High: Black Mid: Black Low: White High: White Mid: White Low: |  |
| One or two hands needed |  |  |
| Use of additional equipment e.g. mirror |  |  |
| Lighting needed e.g. flash or torch |  |  |
| Ease of viewing (e.g. attached viewing screen or additional viewing screen) |  |  |
| Storage of images (SD card or device storage) |  |  |
| Successful capture of fingermark |  |  |
| Security of images |  |  |

Lift Standard Methods Situation 5: Inside an evidence item (Safe)

| Factors of practicability | Results | Observations |
| --- | --- | --- |
| Time Taken | Black High: Black Mid: Black Low: White High: White Mid: White Low: |  |
| One or two hands needed |  |  |
| Use of additional equipment e.g. mirror |  |  |
| Lighting needed e.g. flash or torch |  |  |
| Ease of viewing (e.g. attached viewing screen or additional viewing screen) |  |  |
| Storage of images (SD card or device storage) |  |  |
| Successful capture of fingermark |  |  |
| Security of images |  |  |